AXA/SLB/TechNote/MODA/010.2

JAXA Solar-B Project Office

Solar-B Command Planning and Interface Issues for ISACS-PLN

Appendix 2

Data Recording

Rev. 0.6: 2006 Jun 19

## Revision History

Date		Authors (Affiliation)	Issue

Rev. 0.0		2005 Feb 08	H. Hara (NAOJ/NINS)	All new; not completed

Rev. 0.4		2005 Apr 5	H. Hara (NAOJ/NINS) 	Correction & addition

Rev. 0.5		2005 Apr 13	K. Matsuzaki (ISAS/JAXA) 	Initial Release

Rev. 0.6		2006 Jun 19	K. Matsuzaki (ISAS/JAXA) 	*1

*1: older revision of this document was replaced by the following documents “How to use the Solar-B data recorder with coordination among three instruments (Rev. 1.3, June 19, 2006), Y. Katsukawa (NAOJ)”

This document describes how to coordinate the resource of the Solar-B data recorder among three instruments during their nominal science observing operations.

# 1. General rule

Daily scientific planning should be simple as much as possible unless it does not loose scientific merits. Moreover, since one partition of Solar-B data recorder (described more in details later) is shared among three scientific instruments, the data volume unexpectedly obtained by one instrument should not overwrite the data of the other instruments. Therefore, we will have the following general operation rules.

- Science data taken in a day shall be downlinked during a planning period (nominally one day).

- The MDP function called MDP_TLM_LMT is used to block unexpected over-sized data output from each instrument at MDP (see 6-[2]).

# 2. Data recorder (DR)

The Solar-B data recorder is schematically described in Figure 1.

- The total data volume of the Solar-B DR is 8Gbit.

- There are two partitions in the DR. The nominal partitions are 1Gbit for HK data 　(partition-1) and 7Gbit for science data (partition-2), which are adjustable by command. The HK data is type-I data, including HK1, HK2, AOCS packets. The science data contains not only image data but also instruments’ status packets.

- The science data is downlinked at both Uchinoura (USC) and Svalvard (SVA) stations, but the HK data is nominally down-linked at USC only.

- The DR continuously records the science data to the partition-2 without distinction among SOT, EIS and XRT.

- The data recording works simultaneously with data reproducing for downlink during contact passes.

PT1

RP

WP

1Gbit

Bus

PT2

RP

WP

7Gbit

SOT, EIS, XRT

Telemetry

PT1: Partition-1

PT2: Partition-2

WP: Write pointer

RP: Read pointer

Figure 1: Schematic configuration of the data recorder (DR) and its partitions.

# 3. Planning flowchart

The daily planning of the DR will be performed, by following the flowchart below.

Operation planning of DR reproduction

Calculation of time-averaged data acquisition rates

Allocation of the DR volume to three instruments

Determination of down-link passes

Operation planning of observation tables

Chief planner

(Section 4)

Chief observer

(Section 5)

Confirmation of the data acquisition rate

Chief planner

(Section 6)

Operation planning of MDP_TLM_LMT

Calculation of the data acquisition rates

Preparation of an ASCII file containing the data acquisition rates

Allocation of the downlinked data size to three instruments

Preparation of an ASCII file containing the information of the allocation

Figure 2: Flowchart for the daily planning of the DR.

# 4. Planning of the data rate by a chief planner

A chief planner makes an ASCII format file which contains the information about (1) the allocation of DR volume, (2) the data size down-linked at each contact pass, and (3) the time-averaged data acquisition rate. Its example is shown in Figure 3. This ASCII file is sent to chief observers for their planning of science operation.

(1)

(2)

File: dr_20070401.txt

/* DR VOLUME ALLOCATION */

/*  SOT     EIS     XRT  */

/*(Mbit)  (Mbit)  (Mbit) */

/*-----------------------*/

4900    1050    1050

/*-----------------------*/

/* DATA SIZE DOWNLINKED AT CONTACT PASSES */

/*STA   DATE     TIME    DUR    SOT    EIS    XRT  */

/*               (UT)   (sec) (Mbit) (Mbit) (Mbit) */

/*-------------------------------------------------*/

USC   20070401  091714   596   1668    357    357

SVA   20070401  104104   296    828    177    177

USC   20070401  105513   257    719    154    154

SVA   20070401  121917   313    876    187    187

SVA   20070401  135644   424   1187    254    254

SVA   20070401  153344   526   1472    315    315

SVA   20070401  171028   602   1685    361    361

SVA   20070401  184656   634   1775    380    380

USC   20070401  200620   554   1551    332    332

SVA   20070401  202310   600   1680    360    360

USC   20070401  214312   468   1310    280    280

SVA   20070401  215908   593   1660    355    355

SVA   20070401  233456   593   1660    355    355

SVA   20070402  011045   601   1682    360    360

SVA   20070402  024654   582   1629    349    349

SVA   20070402  042343   550   1540    330    330

SVA   20070402  060123   498   1394    298    298

SVA   20070402  073954   338    946    202    202

USC   20070402  075859   341    954    204    204

SVA   20070402  091859   256    716    153    153

USC   20070402  093338   586   1640    351    351

(1-day total)                 28582   6125   6125

/*-------------------------------------------------*/

/* TIME-AVERAGED DATA ACQUISITION RATE */

/*DATE1   TIME1    DATE2    TIME2     SOT    EIS    XRT  */

/*        (UT)              (UT)    (Mbps) (Mbps) (Mbps) */

/*-------------------------------------------------------*/

20070401  090000  20070401  122917   0.337  0.072  0.072

20070401  122918  20070401  183655   0.197  0.042  0.042

20070401  183656  20070401  220908   0.626  0.134  0.134

20070401  220909  20070401  072953   0.235  0.050  0.050

20070401  072954  20070401  100000   0.530  0.113  0.113

(1-day average)                      0.327  0.070  0.070

/*-------------------------------------------------------*/

(3)

Figure 3: Example of the ASCII file given by the chief planner.

- DR volume allocation for three telescopes

- The data area for science data (partition-2) is shared among SOT, XRT, and EIS. The nominal allocation is 70% (4.9Gbit) for SOT, 15% (1.05Gbit) for XRT, and 15% (1.05Gbit) for EIS. This volume allocation can be changed through a discussion in a weekly/monthly meeting.

- No volume is reserved as margin space in DR. Considering uncertainties in the data size due to compression rate and occurrence of flare observations, a chief observer should make his operation plan which would produce the data to keep the stored data size much below the DR volume allocated to his instrument.

- Data size downlinked at each contact pass

- The chief planner plans the start and stop times of the DR reproduction at each contact pass, based on the information of the contact pass schedule.

- The data size downlinked at each pass is calculated from the downlink rate (4Mbps) and duration of DR reproduction.

- The downlink bandwidth at each pass is shared among SOT, XRT, and EIS data and the data stored in the DR partition is sequentially reproduced starting from the initial position of the read pointer. For planning purpose, the size of downlinked data for each instrument at each pass is defined, although the actual downlinked data size slightly depends on observation sequences of the instruments. The nominal allocation is 70% for SOT, 15% for XRT, and 15% for EIS, which is nominally same as the DR volume allocation. This allocation can be changed through a discussion in a weekly/monthly meeting.

- No margin exists in downlink bandwidth. Considering uncertainties in the data size due to compression rate and occurrence of flare observations, a chief observer should make his operation plan in which the acquired data can be downlinked completely within its scheduled downlinked size.

- Time-averaged data acquisition rates

- The chief planner calculates the time-averaged data acquisition rates for each instrument. The data rates shall be used as a reference to help operation planning by a chief observer. The entire data can be completely downlinked when an instrument obtains science data with this data rate.

- Higher data rate is expected when an orbit contains both USC and SVA passes. One day period can be divided into four periods, USC (evening) + SVA, SVA only, USC (morning) + SVA, and SVA only. The time-averaged data rates are calculated in each period (see Figure 4).

SVA

USC

Figure 4: Contact passes and data acquisition rate for the three instruments as an example. Higher data rates are expected during the periods which have both SVA and USC passes. The solid lines show the data acquisition rate during the divided periods, and the dotted lines show the rate averaged over 1 day.

# 5. Planning of the data rate by chief observers

- Calculation of the data acquisition rate

  - The chief observer of each telescope makes operation plan and observation tables, with considering the DR volume allocation and the data size downlinked at contact passes given by the chief planner (Figure 3). The time-averaged data rates in the ASCII file can be used as a reference for his planning.

  - After making operation plan, the chief observer calculates the data acquisition rates expected from the observation tables as a function of time.

  - The chief observer makes an ASCII format file which contains the data acquisition rates as a function of time, and sends it to the chief planner. Its example in the case of SOT is shown in Figure 5.

File: dr_sot_20070401.txt

/* SOT DATA RATE */

/*DATE1   TIME1    DATE2    TIME2     SOT  */

/*        (UT)              (UT)    (Mbps) */

/*-----------------------------------------*/

20070401  090000  20070401  105959   0.515

20070401  110000  20070401  172959   0.102

20070401  173000  20070401  205859   0.620

20070401  205900  20070402  025959   0.241

20070402  030000  20070402  051234   0.402

20070402  041235  20070402  093500   0.150

/*-----------------------------------------*/

Figure 5: Example of the ASCII file given by the chief observer.

- The chief observer should check whether the data acquisition rates meet the following rules (see Figure 6).

- The data can be downlinked until the next planning period.

- The data size stored in DR is within the size allocated to each instrument.

Since no margin is prepared for the DR volume allocation and the downlinked data size given by the chief planner, the chief observer should have enough margins in the data rates taking into account their uncertainties (e.g. compression efficiency, flare observation).

(1) Chief observer should confirm the data size in DR is always below this limit.

Data size stored in DR

(2) Chief observer should confirm the data is downlinked until the next planning.

Figure 6: Example to show the data size stored in DR as a function of time. The chief observer of each instrument has to check (1) the data size in DR is always below its allocated size and (2) the data is downlinked until the next planning period.

# 6. Confirmation of the data acquisition rate by the chief planner

- The chief planner collects the ASCII files including the data acquisition rates from the chief observers, and double-checks the following rules.

- The data can be down-linked until the next planning period.

- The data size stored in DR is within the capacity.

- MDP has capability, called MDP_TLM_LMT, which pauses the transfer of a telescope’s data to DR (DHU), when the cumulative data size from the telescope exceeds its limit. This finally results in sending BUSY signal to the telescope to stop the observation. This capability is used so that the other telescopes would not be affected by unexpected over–sized data output from one telescope. The chief planner makes a plan to operate MDP_TLM_LMT with the following manner based on the data rates given by the chief observers (Figure 7).

  - MDP_{SOT, XRT, EIS}_TLM_CNT_CLR commands are issued just after each contact pass to clear the counter of the cumulative data size for each of the telescopes.

  - MDP_{SOT, XRT, EIS}_TLM_LMT_SET commands are issued after the MDP_{SOT, XRT, EIS}_TLM_CNT_CLR commands. The data size set by the command is calculated for each instrument as follows:

TLM_LMT = (Data rate)  (Time interval between TLM_CNT_CLR).

The data rates given by the chief observers are used in the above calculation.

pass-1

pass-2

TLM_CNT_CLR

TLM_LMT_SET

Data size in DR

time

TLM_CNT_CLR

TLM_LMT_SET

TLM_LMT

Figure 7: Operation of MDP_TLM_LMT at contact passes.