Monica Bobra

6 April 2006

Data Rate Simulator

In this document, I will outline how to calculate the memory and time necessary for XRT to take any given single image using either the Fast or Safe Mode Exposure.

Due to overlapping mechanical and electrical processes, there are many ways to define the beginning or end of an image. In this calculation, the beginning of an image is defined as when the Filter Wheel 2 movement or the CCD readout (whichever takes longer) for the previous image stops. Note that with this methodology, the first-ever image taken will be longer than the rest of the images.

There are X variables in the calculation: MEN NUMBER, FOV, CCD READ TIME, SHUTTER PREPOSITIONING, SWEEPOUT DELAY, COMPRESSION, RESOLUTION, and MINIMUM TIME. The MDP Expose Busy command is ignored in this calculation as it always overlaps with some other process.  Shutter pre-positioning and sweepout delay are assumed to occur for every image taken. I decided to do this to be conservative and because the error in estimating CCD_EXPOSE is far greater than the time it takes to pre-position the shutter or perform a sweepout delay.

The variable ‘switchmode’ refers to Fast or Safe Mode Exposure. If If XRT is operating in Fast Mode, the variable ‘switchmode’ equals 1. If XRT is operating in Safe Mode, the variable ‘switchmode’ equals 0.

I present the IDL code in its entirety, interspersed with comments.

CODE:

MEN NUMBER: This number calculates CCD_EXPOSE for any given combination of thin, middle and wide slits on the focal plane shutter. The variable ‘men’ is pulled from the ‘MDP Exposure No. (MEN)’ entry in the Parameter Table. Note that the units for duration are ms*(second/ms)=seconds.

; nm_factor=57.9 ms

; nm_delay=79.9 ms

; wide_factor=48 ms

if men eq 0 then pass eq 1 and duration eq 1.024/100

if men eq 1 then pass eq 2 and duration eq 1.024/100

if men eq 2 then pass eq 3 and duration eq 1.024/100

if men eq 3 then pass eq 4 and duration eq 1.024/100

if men eq 4 then pass eq 5 and duration eq 1.024/100

if men eq 5 then pass eq 1 and duration eq 8.192/100

if men eq 6 then pass eq 5 and duration eq 8.192/100

if men eq 7 then pass eq 2 and duration eq 8.192/100

if men eq 8 then pass eq 3 and duration eq 8.192/100

if men eq 9 then pass eq 4 and duration eq 8.192/100

if men eq 10 then pass eq 1 and duration eq 45.056/100

if men eq 11 then pass eq 1 and duration eq 64.512/100

if men eq 12 then pass eq 1 and duration eq 88.064/100

if men eq 13 then pass eq 1 and duration eq 128./100

if men eq 14 then pass eq 1 and duration eq 181.248/100

if men eq 15 then pass eq 1 and duration eq 256./100

if men eq 16 then pass eq 1 and duration eq 362.496/100

if men eq 17 then pass eq 1 and duration eq 512./100

if men eq 18 then pass eq 1 and duration eq 723.968/100

if men eq 19 then pass eq 1 and duration eq 1024./100

if men eq 20 then pass eq 1 and duration eq 1443.84/100

if men eq 21 then pass eq 1 and duration eq 2048./100

if men eq 22 then pass eq 1 and duration eq 2897.92/100

if men eq 23 then pass eq 1 and duration eq 4096./100

if men eq 24 then pass eq 1 and duration eq 5795.84/100

if men eq 25 then pass eq 1 and duration eq 8192./100

if men eq 26 then pass eq 1 and duration eq 11571.2/100

if men eq 27 then pass eq 1 and duration eq 16384./100

if men eq 28 then pass eq 1 and duration eq 23142.4/100

if men eq 29 then pass eq 1 and duration eq 32768./100

if men eq 30 then pass eq 1 and duration eq 46284.8/100

if men eq 31 then pass eq 1 and duration eq 65536.0/100

if men eq 32 then pass eq 1 and duration eq 65536.0/100

if men eq 33 then pass eq 1 and duration eq 65536.0/100

if men eq 34 then pass eq 1 and duration eq 65536.0/100

if men eq 35 then pass eq 1 and duration eq 65536.0/100

if men le 9 then step=(pass*.579)+(pass-1)*.799

if men ge 10 then step=(1.024*duration)+.48

FOV: This defines the field of view based on the variables ‘hsize’ and ‘vsize’ in the ROI table.

xpixels=1

ypixels=1

if hsize eq 1 then xpixels eq 64

if hsize eq 2 then xpixels eq 128

if hsize eq 3 then xpixels eq 192

if hsize eq 4 then xpixels eq 256

if hsize eq 6 then xpixels eq 384

if hsize eq 8 then xpixels eq 512

if hsize eq 12 then xpixels eq 768

if hsize eq 16 then xpixels eq 1024

if hsize eq 24 then xpixels eq 1536

if hsize eq 32 then xpixels eq 2048

if vsize eq 1 then ypixels eq 64

if vsize eq 2 then ypixels eq 128

if vsize eq 3 then ypixels eq 192

if vsize eq 4 then ypixels eq 256

if vsize eq 6 then ypixels eq 384

if vsize eq 8 then ypixels eq 512

if vsize eq 12 then ypixels eq 768

if vsize eq 16 then ypixels eq 1024

if vsize eq 24 then ypixels eq 1536

if vsize eq 32 then ypixels eq 2048

pixels=xpixels*ypixels

CCD READ TIME: This defines the amount of time it takes to read the CCD and is calculated using various algorithms obtained from Kano-san. He says: There are three phases in the CCD read-out time: Pre-Dump, Read-Out of FOV (in the narrow sense), and Post-Dump.  The durations for each phases are shown below.

1) Pre-Dump

Duration to sweep out the signals below the FOV.

t1= v/rv

2) Read-Out of FOV (in the narrow sense)

Duration to read out the signal in the FOV.

t2=(bin/rv + 2176/rh)*(wv/bin)

3) Post-Dump

Duration to sweep out the dark signal during the readout.

t3= 2112/rv

The CCD read-out time is a total of them: t = t1 + t2 + t3

Variables:

h: start horizontal address of FOV. First column is 0.

v: start vertical address of FOV. First line is 0.

wh: horizontal width of FOV.

wv: vertical   width of FOV.

bin: chip sum number

rh: the rate of horizontal transfer. 500,000pixel/sec

rv: the rate of vertical   transfer. 15,625line/sec

To read more about CCD READ TIME, see APPENDIX 1.

h=2048.0-(1024.0+xpixels/2)

v=2048.0-(1024.0+ypixels/2)

a1=v/15625.0

a2=(res/15625.0+0.00435200)*(ypixels/res)

a3=0.135168

t=a1+a2+a3

VISIBLE LIGHT SHUTTER: This defines the amount of time it takes to open the visible light shutter. If the variable ‘vls’ equals 0, the visible light shutter is closed; if the variable ‘vls’ equals 1, the visible light shutter is open. The value for the ‘vls’ variable comes from the ‘Aspect Shutter’ line in the Parameter table.

t1=1.0

if switchmode eq 0 and vls eq 0 then t1=0.

if switchmode eq 0 and vls eq 1 then t1=1.2

if switchmode eq 1 and vls eq 0 then t1=0.

if switchmode eq 1 and vls eq 1 then t1=1.2

SHUTTER PRE-POSITIONING: This is the amount of time it takes to chance the focal plane shutter from one position to another. Shutter pre-positioning is always 120 ms and is assumed to always occur.

t4=.12

SWEEPOUT DELAY: This is a fixed delay that occurs if there has been any shutter pre-positioning or filter wheel motion. The sweepout delay is always 250 ms and it is assumed to always occur.

t2=.25

FILTER WHEEL: This is the amount of time it takes to move the filter wheels from any given position (A) to any given other position (B). The variable ‘filter_[x]’ comes from the ‘Def. Filter A/B’ entry in the Parameter table. Since ‘A/B’ is a string, I had to parce it to recover the filter wheel numbers. In this example, ‘prev_filter_[x]’ is defined as the filter position for the first (or previous) image. In this case, each filter wheel step takes 1.5 seconds so I just take the absolute value of the current filter position subtracted from the previous filter position and multiply that quantity by 1.5 to get the time. However, I assume that for all cases in which either filter wheel must go from position 1 to 5 (thereby going through 0), the filter wheel will take the longer path.

filter_a=fix(strmid(filter,0,1))

filter_b=fix(strmid(filter,2,1))

prev_filter_a=0

prev_filter_b=1

t3_a=1.0

t3_b=1.0

if prev_filter_a eq 1 and filter_a eq 5 then t3_a=4.5 $

else t3=(abs(prev_filter_a-filter_a))*1.5

if prev_filter_b eq 1 and filter_b eq 5 then t3_b=4.5 $

else t3=(abs(prev_filter_b-filter_b))*1.5

t3=(t3_a)+(t3_b)

prev_filter_a=filter_a ; pass variable name

prev_filter_b=filter_b ; pass variable name

COMPRESSION: This determines how compressed each image is. The variable ‘cmp’ is pulled from the ‘CMP Code’ entry in the Sequence Table. Note that the calculation is pixels*bits/pixel = bits.  I obtained the following compression codes from Kano-san. To see more about compression codes, see APPENDIX 2.

0x0020(no comp) -   12  bit/pixel
0x0328(DPCM) -  6.0-7.8 bit/pixel

0x0728(Q=98) - <5.4-7.1 bit/pixel

0x0729(Q=90) - <3.0-4.2 bit/pixel

0x072A(Q=75) - <2.4-3.0 bit/pixel

0x072B(Q=50) - <1.8-2.4 bit/pixel

0x072C(Q=95) - <4.5-6.0 bit/pixel

0x072D(Q=92) - <3.6-4.9 bit/pixel

0x072E(Q=85) - <2.8-3.8 bit/pixel

0x072F(Q=65) - <2.2-2.8 bit/pixel.

if cmp eq '0x0020' then pixels=pixels*12

if cmp eq '0x0328' then pixels=pixels*6.9

if cmp eq '0x0728' then pixels=pixels*6.25

if cmp eq '0x0729' then pixels=pixels*3.6

if cmp eq '0x072A' then pixels=pixels*2.7

if cmp eq '0x072B' then pixels=pixels*2.1

if cmp eq '0x072C' then pixels=pixels*5.25

if cmp eq '0x072D' then pixels=pixels*4.25

if cmp eq '0x072E' then pixels=pixels*3.3

if cmp eq '0x072F' then pixels=pixels*2.5

RESOLUTION: This determines if the pixels are rebinned. The variable ‘res’ is pulled from the ‘Resolution: Chip Sum’ entry in the Parameter table. Note that the variable ‘pixels’ already has the units of bits.

if res eq 0 then pixels=pixels

if res eq 1 then pixels=pixels/4

if res eq 2 then pixels=pixels/16

if res eq 3 then pixels=pixels/64

FAST MODE/SAFE MODE SWITCH: If XRT is operating in Fast Mode, the variable ‘switchmode’ equals 1. If XRT is operating in Safe Mode, the variable ‘switchmode’ equals 0.

if switchmode eq 0 $

then time_seconds=t3+t4+((t1)/2)+t2+step+t

if switchmode eq 1 then q=((t1)/2)+t3

if q le t then time_seconds=((t1)/2)+t4+t2+(t)+step

if q gt t then time_seconds=((t1)/2)+t4+t2+(q)+step

pixels=bits

bytes=bits*8

kilobytes=bytes*1024

MINIMUM TIME: This ensures that the observation table runs for a minimum amount of time. The variable ‘mintime’ comes from the ‘Interval Time’ entry in the Sequence table. There is also a minimum time given in the Observation and Subroutine tables that need to be taken into an account when calculating the time for a series of images.

if mintime ge time_seconds then mintime=time_seconds

if mintime lt time_seconds then time_seconds=time_seconds

print, "time_seconds equals the amount of time in seconds. $

kilobytes equals the amount of memory in kbytes."

print, time_seconds

print, kilobytes

end

APPENDIX 1: CCD READ TIME EXAMPLES (from Kano-san)

Example1: Full frame image with 2"-resolution.

h=0,v=0,wh=2048,wv=2048,bin=2

t1=0/15625=0sec

t2=(2/15625+2176/500000)*(2048/2)=4.58752sec

t3=2112/15625=0.135168

t = 4.723 sec

Example2: Central 1kx1k field-of-view with 1"-resolution

h=512,v=512,wh=1024,wv=1024,bin=1

t1=512/15625=0.032768sec

t2=(1/15625+2176/500000)*(1024/1)=4.521984sec

t3=2112/15625=0.135168

t = 4.690 sec

Example3: CAL data for CCD

The CAL data for CCD has a special format.

h=N/A, v=0, wh=2176, wv=2112, bin=1

t1=0/15625=0sec

t2=(1/15625+2176/500000)*(2112/1)=9.326592sec

t3=2112/15625=0.135168

t = 9.462 sec

APPENDIX 2: COMPRESSION CODES (from Kano-san)

The Q-tables have been revised, and currently their Q-factors are:

CMP=0x0728   Q=98

0x0729   Q=90

0x072A   Q=75

0x072B   Q=50

0x072C   Q=95

0x072D   Q=92

0x072E   Q=85

0x072F   Q=65.

Here I will roughly estimate the compression percentage by interpolating  the above Q-factors into the the results described in Sakao's e-mail.

0x0728(Q=98) - <45-59 %

0x0729(Q=90) - <25-35 %

0x072A(Q=75) - <20-25 %

0x072B(Q=50) - <15-20 %

0x072C(Q=95) - <37-50 %

0x072D(Q=92) - <30-41 %

0x072E(Q=85) - <23-31 %

0x072F(Q=65) - <18-23 %

For XRT, we can use the other two CMP codes; 0x0020 = no compression and 0x0328 = DPCM lossless. The compression percentage by DPCM is almost the same with that by JPEG with Q=100. The result for Q=100 was described in Sakao's e-mail. Therefor, I will write their percentages in the same format with those by JPEG.

0x0020(no comp) - 100 %

0x0328(DPCM) -  50-65 %

If you prefer bits/pixel, the above percentages are:

0x0020(no comp) -   12  bit/pixel
0x0328(DPCM) -  6.0-7.8 bit/pixel

0x0728(Q=98) - <5.4-7.1 bit/pixel

0x0729(Q=90) - <3.0-4.2 bit/pixel

0x072A(Q=75) - <2.4-3.0 bit/pixel

0x072B(Q=50) - <1.8-2.4 bit/pixel

0x072C(Q=95) - <4.5-6.0 bit/pixel

0x072D(Q=92) - <3.6-4.9 bit/pixel

0x072E(Q=85) - <2.8-3.8 bit/pixel

0x072F(Q=65) - <2.2-2.8 bit/pixel.